Report 99.460

20 August 1999

File: T/8/1/3

[99.460.awb]

Report to the Passenger Transport Committee

By Tony Brennand, Manager Transport Policy

Park and Ride Update

1.	Purpose

To update the Committee on the current levels of occupancy at the Region’s railway park and ride sites and to recommend further projects be added to the Minor Passenger Transport Infrastructure Priority List.

2.	Background

The Region’s passenger rail network is supported by park and ride sites which extends the accessibility provided by the rail network beyond the immediate environs of the rail corridor.  Approximately 20 percent of the passengers who catch the train during the morning peak at any particular station would make use of a park and ride facility. This figure does vary depending on the level of service provided at the station, geographic factors and the capacity of carparking at each station.

Regular counts of carparking are undertaken at each of the stations so that the occupancy levels of the carparks compared to capacity can be assessed.  Historic trends have shown a strong growing demand for carparking at the station carparks.

3.	Comment

…		The table in Attachment A shows carparking characteristics at each station.  Figures are provided for the carpark capacity in accordance to normal carpark standards, the recent average occupancy of the carpark (as at 30 June 1999) and the total occupancy expressed as a percent of site capacity.

It should be noted that in some cases the carpark capacity is a nominal figure.  This is because the carpark boundaries are not precisely defined and all the available space has yet to be formed into formal carparks.  These sites are denoted by a ‘+’ in the available spaces column.

A number of sites have occupancy levels that exceed capacity.  At these sites cars are parked in ways that do not meet carpark standards and access in and out of the carpark by other patrons can be compromised.  It has also been observed that at these carparks where demand exceeds capacity a number of patrons will park in local streets near the station.  This has not been allowed for in the statistics provided in Attachment A, except where a designated area of on street carparking is formally recognised as part of the park and ride facility.  It is likely, then, where demand exceeds capacity, that actual demand could be greater than that indicated by the occupancy levels.  Parking on local streets, can in some circumstances, create problems for the local community.

The table in Attachment A shows that the Region has in the order of 2,740 spaces as part of formal rail park and ride facilities.  The total demand for parking is at least 2,460 spaces which represents an average demand to capacity ratio of at least 90 percent.

The table in Attachment A identifies that the demand equals or exceeds capacity at the following stations:

- Waterloo

- Woburn

- Petone

- Featherston

- Carterton

- Paraparaumu

- Simla Crescent

- Khandallah

- Raroa

- Johnsonville

- Awarua Street

These sites would appear as priority sites for carpark expansion.

The table in Attachment A identifies that demand exceeds 95% of capacity (but less than 100%) at the following stations:

- Upper Hutt

- Trentham

- Redwood

These sites would appear as second priority sites for carpark expansion.

The priority sites have been evaluated as follows:

…	(i)	Waterloo - Proposals have been developed and appear on the Minor Projects Priority List shown in Attachment B.  The project is committed.  A further proposal to address medium term needs providing 45 spaces at $150,000 has also been identified.

(ii)	Woburn – It is estimated that another 30 spaces should be provided at a cost of approximately $60,000.  As there is some available on  street carparking near the station this is not regarded as an urgent priority.

(iii)	Petone – Provision for another 20 spaces has been made on the Minor Projects Priority List.  However, this is regarded as only a short term provision as current growth of parking will quickly consume these spaces.  Provision for medium term needs should be considered but options are likely to be expensive and will require some land purchase.  A sum of  $300,000 should be allocated for medium term needs as an indicative estimate.

(iv)	Featherston – Options exist for this site and it is estimated that a further 25 spaces should be provided to meet short to medium term needs.  This is estimated to cost $50,000.

(v)	Carterton – Another 15 spaces are required to meet medium term needs. $30,000 should be allocated to provide for these spaces.  However, as there is available on street parking this is not regarded as an urgent priority.

(vi)	Paraparaumu – Provision has been made in the minor projects list for further land purchase.  This is not regarded as an urgent priority as there is land that has been recently purchased that will become available in the near future.  It is also recognised that the provision of a station at Raumati and the extension of regular services beyond Paraparaumu will take pressure off carparking at Paraparaumu although the increased frequency of services will reduce the impact of this relief.

(vii)	Simla Crescent – Options exist to cut into a bank which extends the existing facility.  If this work is to occur it is more economic to put in a sizeable increment of parking rather than a series of small increments of 2 or 3 spaces. Ten spaces could cost $35,000 to construct.  As available spaces exist on the street this is not regarded as an urgent priority.

(viii)	Khandallah – No obvious options exist for this site.

(ix)	Raroa – A pavement already exists although spaces are not marked up.  There are on street spaces available.  This is not regarded as a priority.

(x)	Johnsonville – Options exist for carpark expansion.  Approximately 20 spaces should be provided to address medium term needs.  $50,000 should be allocated for this purpose.

(xi)	Awarua Street – Options exist to provide for another 4 to 5 spaces.  $10,000 should be allocated for this purpose.

(xii)	Upper Hutt – Provision has already been made for this site in the minor projects list.

(xiii)	Trentham – Provision has already been made for this site in the minor projects list.

(xiv)	Redwood – Another 20 spaces should be provided to meet medium term needs.  This is estimated to cost $40,000.  As this site is not at capacity this proposal is not an urgent priority.

…		On the basis of the above analysis it is proposed that the Minor Projects Priority List (refer Attachment B) be revised as shown in Attachment C.

4.	Recommendation

- That the information contained in this report be noted.

- That the Minor Projects Priority List be revised as shown in Attachment C.

Report prepared by:					Approved for submission by:

TONY BRENNAND	DAVE WATSON

Manager, Transport Policy	Divisional Manager, Transport

Attachments (3)

# Railway Station Parking Levels

| Site | Available Spaces | Average Occupancy | Percent Occupied |
| --- | --- | --- | --- |
| Upper Hutt Wallaceville Trentham Silverstream Taita Waterloo Woburn Petone Melling | 156 42 57 60 127 455 112 143 185 | 148 31 55 56 91 488 112 152 123 | 95 74 97 93 72 107 100 106 66 |
| Featherston Woodside Carterton Solway Masterton | 70 42 44 10+ 65+ | 71 31 44 9 56 | 101 74 100 90 86 |
| Takapu Road Redwood Mellville Tawa Duncan Tawa Porirua Paremata Mana Plimmerton Pukerua Bay/Muri Paekakariki Paraparaumu Waikanae | 64 97 30+ 10 250+ 162 20 35+ 25+ 40 250 50 | 60 92 12 8 195 135 16 25 2 27 260 30 | 94 95 40 80 79 83 80 71 8 68 104 60 |
| Crofton Downs Ngaio Simlar Crescent Khandallah Raroa Johnsonville Awarua Street | 44 25 6 7 8 43 4 | 40 21 6 7 8 46 4 | 91 84 100 100 100 107 100 |
|  | 2738+ | 2461 | 90 |

Priority List for Minor Passenger Transport Projects Infrastructure Items

| - Waterloo Carpark Extension Committed | $150,000 |
| --- | --- |
| - Wellington Station Subway Development | $940,000 |
| - Sundry Bus Shelters | $70,000 |
| - Karori Bus Lane Stage 1 Not Ready to Proceed | $95,000 |
| - Wellington Station Pedestrian Shelter | $450,000 |
| - Security Information System Stage 1 | $150,000 |
| - Porirua Carpark Improvements Stage 1 | $135,000 |
| - Bunny-Featherston Pedestrian Shelter | $400,000 |
| - Porirua Transport Centre Stage 2 | $50,000 |
| - Trentham Commuter Carpark Extension | $50,000 |
| - Petone Commuter Carpark Extension | $40,000 |
| - Bunny Street Refuge and Kerb Extension | $200,000 |
| - Pedestrian Canopy Waterloo/Whitmore Street | $165,000 |
| - Wellington Railway Station Enhancements | $255,000 |
| - Karori Bus Lane Stage 2 Not Ready to Proceed | $70,000 |
| - Paraparaumu Carpark Land Purchase Stage 2 | $120,000 |
| - Lambton Quay Pedestrian Canopy | $250,000 |
| - Solway Station Carpark Development | $18,000 |
| - Porirua Carpark Improvements Stage 2 | $135,000 |
| - Porirua Transport Centre Stage 3 | $50,000 |
| - Upper Hutt Carpark Expansion | $150,000 |
| - Lambton Terminal Upgrade | $205,000 |
| - Countdown Shelter Stage 2 | $50,000 |
| - Paraparaumu Carpark Enhancements | $200,000 |
| - Solway Carpark Enhancements | $12,000 |
| - Featherston Carpark Enhancements | $15,000 |
| - Wallaceville Carpark Enhancements | $15,000 |
| - Waikanae Carpark Enhancements | $30,000 |
| - Security Information System Stage 2 | $150,000 |
| - Paekakariki Overbridge Development | $120,000 |
| - Paekakariki Carpark Development | $50,000 |
|  | $4,790,000 |

Priority List for Minor Passenger Transport Projects Infrastructure Items

| - Waterloo Carpark Extension Committed | $150,000 |
| --- | --- |
| - Wellington Station Subway Development | $940,000 |
| - Sundry Bus Shelters | $70,000 |
| - Karori Bus Lane Stage 1 Not Ready to Proceed | $95,000 |
| - Wellington Station Pedestrian Shelter | $450,000 |
| - Johnsonville Carpark Extension | $50,000 |
| - Security Information System Stage 1 | $150,000 |
| - Porirua Carpark Improvements Stage 1 | $135,000 |
| - Bunny-Featherston Pedestrian Shelter | $400,000 |
| - Porirua Transport Centre Stage 2 | $50,000 |
| - Petone Commuter Carpark Extension | $40,000 |
| - Bunny Street Refuge and Kerb Extension | $200,000 |
| - Pedestrian Canopy Waterloo/Whitmore Street | $165,000 |
| - Wellington Railway Station Enhancements | $255,000 |
| - Featherston Carpark Expansion | $50,000 |
| - Awarua Street Carpark Expansion | $10,000 |
| - Karori Bus Lane Stage 2 Not Ready to Proceed | $70,000 |
| - Paraparaumu Carpark Land Purchase Stage 2 | $120,000 |
| - Waterloo Carpark Expansion | $160,000 |
| - Lambton Quay Pedestrian Canopy | $250,000 |
| - Petone Carpark Expansion | $300,000 |
| - Solway Station Carpark Development | $18,000 |
| - Woburn Carpark Expansion | $60,000 |
| - Porirua Carpark Improvements Stage 2 | $135,000 |
| - Carterton Carpark Expansion | $30,000 |
| - Porirua Transport Centre Stage 3 | $50,000 |
| - Simla Crescent Carpark Expansion | $35,000 |
| - Upper Hutt Carpark Expansion | $150,000 |
| - Lambton Terminal Upgrade | $205,000 |
| - Countdown Shelter Stage 2 | $50,000 |
| - Paraparaumu Carpark Enhancements | $200,000 |
| - Redwood Carpark Expansion | $40,000 |
| - Trentham Carpark Expansion | $50,000 |
| - Solway Carpark Enhancements | $12,000 |
| - Featherston Carpark Enhancements | $15,000 |
| - Wallaceville Carpark Enhancements | $15,000 |
| - Waikanae Carpark Enhancements | $30,000 |
| - Security Information System Stage 2 | $150,000 |
| - Paekakariki Overbridge Development | $120,000) |
| - Paekakariki Carpark Development | $50,000 |
|  | $5,475,000 |